XJW型橡胶冷喂料挤出机系列

Type XJW cold- feed rubber extruder series

1、用途及说明 Use introduction

XJW型冷喂料橡胶挤出机，主要适用于生产电线、电缆绝缘层和护套层。如更换其他形式的机头及相应的辅机，可挤制胶管、胶棒等其它异型橡胶制品的生产。该机可根据用户需要配置直流电机、变频电机及其他调速形式的电机驱动。选用硬齿面齿轮减速箱。温控方式为手动或自动控制，（自行选定其中一种形式）。该机具有传动平稳，性能可靠，质量稳定，噪音小，维护方便，操作简便，造型美观等特点。

Type XJWY cold-feed rubber extruder is new design type of rubber extruder. It is applicable for extrusion of insulation and sheath for wire and cable..If  exchange of various machine-head and cooperating auxiliary equipment , it shall extrude tube , stick and another  special rubber products. It is disposed of variable types driver, for example dc- driver or motor with eddy current clutch and another driver.,  retarded  motor with hard surface of gear. Control temperature is application for manual. Though both metal thermometer and thermorelay or hot-resistance .Customer can select willfully. The equipment is provided with stable drive, reliable performance ,low noise , easy operating, external form artistic.

2、主要技术参数The main technical characteristics

| 型号 Type Name of product | XJW45 | XJW50 | XJW65 | XJW70 | XJW85 | XJW90 | XJW120 | XJW150 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 螺杆直径 Diameter of screw （mm） | 45 | 50 | 65 | 70 | 85 | 90 | 120 | 150 |
| 螺杆长径比Length/diameter （L/D） | 12 | 12 | 12(14 16) | 12(14 16) | 10 | 14(16) | 12(14 16) | 14(16) |
| 螺杆转速 Rotational speed of screw （r/min） | 5.3~80 | 5.3~80 | 6.5~65 | 6.5~65 | 5~50 | 6~60 | 5~45 | 3.5~35 |
| 挤出线芯直径Extruded diameter of core（mm） | 0.75~2.75 | 0.75~2.75 | 1~14 | 1~14 | 4~22 | 6~25 | 13~40 | 25~55 |
| 挤出绝缘直径Extruded diameter of insulation（mm） | 2~5 | 2~5 | 2~15 | 2~15 | 8~26 | 10~30 | 18~50 | 30~65 |
| 护套最大直径Extruded diameter of sheath（mm） | ≤6 | ≤6 | ≤18 | ≤20 | ≤28 | ≤36 | ≤70 | ≤90 |
| 最大生产能力Product capacity(max)（㎏/h） | 35 | 46 | 80 | 130 | 130 | 180 | 320 | 480 |
| 驱动功率 Drive power （KW） | 11 | 11 | 22 | 30 | 30 | 55 | 90 | 132 |
| 机器中心高 Central height of min machine （mm） | 850 | 850 | 1000 | 1000 | 1000 | 1000 | 1150 | 1200 |
| 外形尺寸：约 Dimension of external form （长×宽×高）(appr.)（m） Dimension of external form （长×宽×高）(appr.) | 1.4×1.3×1.1 | 1.4×1.3×1.1 | 1.7×1.4 ×1.4 | 1.9×1.4 ×1.4 | 1.9×1.9×1.3 | 2.8×1.8×1.6 | 3.7×2.4 ×1.6 | 4.1×2.5 ×1.7 |
| 机器重量：约 Mass of machine( appr.)（T） | 1.2 | 1.2 | 2.5 | 2.6 | 3 | 3.5 | 5.5 | 6.5 |