XJW型橡套电缆连续硫化生产线

Type XJW rubber extruding  CV-line for cable

1、适用范围：Suitable scope

该生产线适用于电线、电缆生产0.3KV～10KV中低

压等级的橡胶绝缘、橡胶护套的软电线、电缆，并连续硫化生产的设备。

The product line is applicable for medium-low voltage 0.3-10KV cold-feed rubber insulation and sheath of flexible cable, and continual cure.

2、设备的组成：The composition of machine

该生产线主要由放线装置——导轮——送线涨力控制——过粉箱冷喂料橡胶挤出机

——上密封装置——进汽管——硫化管道及调整支架——下密封装置——不锈钢集水箱

——（冷却水槽）——牵引机——收排线架——电器控制柜等设备组成。

The composition of the machine have below elements: dereeler, frame of accumulator for pay-off, guide roller, tension control for pay-off(haul-off), wire guide, master  machine of extruder, condensing linking tube, tube of admission steam ,curing tube and adjustable support, water seal at the back, collect water box for stainless steel, cooling water both, drawing machine, balance  system  for  water and steam ,frame of take-up accumulator,  distributor carriage ,electrical control system etc.

| 号Type 产品名称Name of product | XJW50 | XJW65 | XJW70 | XJW90 | XJW120 | XJW120 | XJW150 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 螺杆直径 Diameter of screw （mm） | 50 | 65 | 70 | 90 | 120 | 120 | 150 |
| 螺杆长径比 （L/D）Length/diameter | 12 | 12、14、16 | 12、14、16 | 14 | 12、14 | 16 | 14、16 |
| 挤出绝缘直径Extruded diameter of insulation（mm） | 2~5 | 2~15 | 8~26 | 10~30 | 18~50 | 25~60 | 30~65 |
| 护套最大直径Extruded diameter of sheath (max)（mm） | ≤6 | ≤15 | ≤15 | ≤36 | ≤50 | ≤60 | ≤70 |
| 最大生产能力Product capacity(max)（㎏/h） | 46 | 80 | 130 | 180 | 320 | 360 | 480 |
| 驱动功率 Drive power （KW） | 11 | 22 | 30 | 55 | 90 | 90 | 132 |
| 机器中心高height of min machine （mm） | 1000 | 1000 | 1000 | 1000 | 1150 | 1150 | 1200 |
| 螺杆、套筒材质Material of screw and sleeve | 38CrMoAlA | 38CrMoAlA | 38CrMoAlA | 38CrMoAlA | 38CrMoAlA | 38CrMoAlA | 38CrMoAlA |
| 加热区数quantity of heat zone | 5 | 5 | 5 | 5 | 5 | 6 | 6 |
| 硫化管长度Length of cure tube (m) | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| 放线架specification of frame of accumulator for pay-off | 630 | 630 | 630 | φ1600 | Φ1250/Φ2000 | Φ1250/Φ2000 | φ1250／φ2500 |
| 牵引型式drawing type | 500立式Type vertical | 630双轮Dual drum | 630双轮Dual drum | 955双轮Dual drum | 2000kg履带Caterpillar-type | 2000kg履带Caterpillar-type | 3200Kg履带Caterpillar-type |
| 收排线架规格specification of frame for pay-off and take-up | 630双盘Dual drum | 630双盘（Φ1250）Dual drum | 630双盘（Φ1250）Dual drum | φ1600 | Φ1250/Φ2500 | Φ1250/Φ2500 | φ2000／φ3150 |

注：双机共挤时（生产双层、双色）可根据客户具体要求配置不同机型，

硫化管道长度及安装形式可根据客户要求调整