SP-型塑料挤出机系列

Type SP plastic extruder series

- 用途及说明：Use and introduction

SP型单螺杆塑料挤出机主要适用于聚乙烯、聚氯乙烯及聚酰胺（尼龙）等热塑性塑料，用热挤法对导体绝缘层、电缆护套层进行包覆。可调换各种形式的机头及螺杆，与相应的辅机配套使用，即可挤制软、硬聚乙烯、聚氯乙烯、聚丙烯和氟塑料等热塑料原料进行电线绝缘层，电缆护套层。（高温挤出机须特殊定货）该机具有适用性广，传动平稳，性能可靠，质量稳定，噪音小维护方便，操作简便，造型美观等特点。

Type SP general extruder with single screw is applicable for extrusion of plastic products. Exchange of various machine-head and relevant screw , and cooperating auxiliary equipment , it shall extrude tube , stick tape and insulation of wire and cable , that uses flexible and stiff PE, PVC PP, etc. The equipment is provided with stable drive, reliable performance ,low noise , easy operating, external form artistic.

2.主要技术参数：The main technical characteristics

| 型号Type 产品名称 Name of product | SP45/25H 高温High temperature | SP45/25 | SP65/25H 高温High temperature | SP65/25 | SP90/25 | SP120/25 | SP150/25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 螺杆直径Diameter of screw (mm) | 45 | 45 | 65 | 65 | 90 | 120 | 150 |
| 螺杆长径比 (L/D) Length/diameter | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 螺杆转速Rotational speed of screw (r/min) | 35-46 | 30-145 | 6-75 | 6-120 | 10-120 | 5-83 | 6.8-68 |
| 最大挤出直径Extruded diameter (max) (mm) | 5 | 5.5 | 20 | 15 | 45 | 50(75) | 85 |
| 最大生产能力Product capacity(max) (kg/h) | 50 | 65 | 80 | 90(130) | 360 | 550 | 700 |
| 主机中心高Central height of min machine (mm) | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 | 1000 |
| 传动功率 Drive power (kw) | 15 | 15 | 30 | 30 | 90 | 132 | 160 |
| 风机功率 (w) | 180 | 180 | 180 | 180 | 370 | 370 | 850 |
| 加热功率Heat power (kw) | 11.4 | 7.2 | 20 | 14.4 | 25 | 35 | 44.5 |
| 机头加热 Heatting head (kw) | 3 | 1.05 | 3 | 1.8 | 4.8 | 10 | 13 |
| 外形尺寸:约 Dimension of external form（L*W*H）appr. (长*宽*高) (mm) | 1.8*1.2*1.7 | 1.8*1.2*1.7 | 2.5*1.6*1.2 | 2.5*1.6*1.2 | 3.6*2.0*2.0 | 4.4*1.2*2.1 | 5.3*1.3*2.4 |
| 机器重量:约 Mass of machine appr. (T) | 1.2 | 1.2 | 2.2 | 2.2 | 3.2 | 5.8 | 6.5 |